PCR-1000 radio for

UN*X GUI Development

and an OOP Library

Part I: Protocol

A Ghetto.Org Investigation

PolyWog and Javaman

November 16, 1999

Objective:

Create a shared object (or static) library for facilitation of the Icom PCR-1000 UN*Xinterface. Create a GUI and command line interface for the Icom PCR-1000 all band, all mode receiver.

PCR-1000 Serial Protocol:

The PCR-1000 begins each command with a set of plain text codes sent across the serialcable at 9600 baud. Each command is terminated with a CR-LF represented by the HEX codes \x0d and \x0a.

Radio Initialization String:

(Equivalent to sending a power on, and G300 code.)

H101\x0d\x0aG300\x0d\x0a

Basic command codes:

G3NN			- Signal Update

00		- Off (program should poll status)

01		- On (radio sends status automatically when change sent)

02		- Binary Mode (Update off)

03		- Binary Mode (Update on)

H1NN			- Power

00		- Off

00		- On

J40NN			- Volume

00-99		- Level

J41NN 		- Squelch

00-99		- Level

J43NN 		- IF Shift

00-FF		- Level

J45NN			- Automagic Gain Control

00		- Off

01		- On

J46NN	 		- Noise Blanking

00		- Off

01		- On

J47NN			- RF Attenuator

00		- Off

01		- On

J50NN			- VSC

00		- Off

01		- On

J51NN			- Tone Squelch

00		- Off

01-33		- Standard tones [67 Hz - 254.1 Hz]

J4ANN			- <UNKNOWN>

80		- <UNKNOWN>

LD82NN		- Tracking Filter

00		- set to automagic

Frequency Selection Command:

K0 			- Command Initialize

0000000000 		- Frequency (freq's must be padded correctly)

0000050000	- Lower Range (0.050 MHz)

1300000000	- Upper Range (1300 MHz)

0N 			- Mode

00		- Lower Side Band

01		- Upper Side Band

02		- AM

03		- CW

04		- <UNKNOWN>

05		- Narrow FM

06		- Wideband FM

0N 			- Filter

00		- 3 kHz

01		- 6 kHz

02		- 15 kHz

03		- 50 kHz

04		- 230 kHz

00			- Mandatory but arbitrary padding at end.

Sample Output for selecting z-100 WHTZ from NYC's Empire State Building.

-Note this was a totally random selection ;^)

To select: 100.3 FM (WFM) 230kHz filter.

Cmd Out: K00100300000060400

Break down: [K0][0100300000][06][04][00]

Radio Queries:

H1\?			- Is the power on? [Reply: H1NN]

I0\?			- Squelch setting?

I1\?			- Signal strength?

I2\?			- Frequency offset?

I3\?			- DTMF Tone?

G4\?			- Firmware revision?

GE\?			- Country/Region?

Standard Radio Reply:

G0NN			- Standard Reply

00		- OK reply

01		- Bad reply

GDNN			- DSP Present

00		- No

01		- Yes

GENN			- Country Code(s)

09		- US

02		- Euro. /UK

I0NN			- Squelch Status

04		- Closed

07		- Open

I1NN			- Signal Strength

00-FF		- Weak-Strong

I2NN			- Signal Centering

00-7F		- Low

80		- Centered

81-FF		- High

I3NN			- DTMF Tone

00		- None heard

1N		- Heard (N= [0-F] where *=E, and #=F)

Radio functions:

G1NN			- Set baud rate

00		- 300

01		- 1200

02		- 2400

03		- 9600

04		- 19200

05		- 38400

G3NN			- Set autoupdate

00		- Off

01		- On

Protocol Programming Notes:

The radio will at most times reply with Ok or Bad. Should it reply with anything else, then itis an ``actual' reply string from the radio and must be parsed. First we should check to see if it was a reply string without garbage characters. Just strip the CRLF chars and see what is left. Theeasiest way (although not the smartest way) to find out what the reply is for, is to remember what we asked it last, and assume that the reply (if it is not Ok or Bad) is the reply to our question.

In order to understand the radio, you have to know what to expect. The radio seems to send out \n' even when it has nothing to say. So the best thing to do, when deciphering the repliesis to make sure that the length read in is greater than one. Because it will always be, at least, one.

Being patient is also something to hold dear. It may take the radio a grueling .5 seconds to respond to a request. However, it shouldn't (on average) take it more than 3 seconds to reply to any one request. So a timer that triggers SIGALRM may be started and set to 5 seconds. If no reply is received within five seconds, you should assume that something is seriously wrong and attempt either bail, or resend the request.

Kudos to: Javaman for redirecting me to the ``original' pcr.c source that was able to open up theprotocol barrier. And Devi0us for use of the radio.

Next up: [Part II: The Comm]

These are the documented HEX values. Something tells me (if its anything like the squelchand volume) that the value ranges are between 00-99.

For the number padding etc, I have used the standard SQL formatting scheme. N =number, 0=any number or padded with zero if blank, 9 = any number padded or not

We don't care about \x0d\x0a. Since we know that all replies will be 4 bytes in length,then we should chop off the CRLF by null'ing byte MAXLENGTH+1 (or five) ;^)

Use the function strpbrk() sending as the second argument \x0d\x0a 